Issued September 21st, 2012

###### Security White Paper

###### for

Device Manager NX Lite

Solution Support Department

Service and Support Center

Global Marketing Group

Ricoh Company, Ltd.

Document version: 1.0

Notice:

This document may not be reproduced or distributed in whole or in part, for any purpose or in any fashion without the prior written consent of Ricoh Company Limited. Ricoh Company Limited retains the sole discretion to grant or deny consent to any person or party.

Copyright © 2012 by Ricoh Company Ltd.

All product names or product illustrations, including desktop images, used in this document are trademarks, registered trademarks or the property of their respective companies. They are used throughout this book in an informational or editorial fashion only. Ricoh Company, Ltd. does not grant or intend to grant hereby any right to such trademarks or property to any third parties. The use of any trade name or web site is not intended to convey endorsement or any other affiliation with Ricoh products.

Although best efforts were made to prepare this document, Ricoh Company Limited makes no representation or warranties of any kind with regards to the completeness or accuracy of the contents and accepts no liability of any kind including but not limited to performance, merchantability, fitness for any particular purpose, or any losses or damages of any kind caused or alleged to be caused directly or indirectly from this document.

Document version history:

| Version | Date of Issue | Revision |
| --- | --- | --- |
| 1.0 | September, 2012 | 1st Release |

1. Introduction	4

2. System	5

2.1. Internet communication diagram	5

2.2. Intranet communication diagram	6

3. Data Flow	7

3.1. Internet communication data flow	7

3.2. Intranet communication data flow	7

4. Access account	9

4.1. SNMP Access Account	9

4.2. Web Access Account	9

5. Protocols and Ports	10

5.1. Discovery	10

5.2. Device Polling (Status)	10

5.3. Device Polling (Tray/Toner Ink)	10

5.4. Device Polling (Counter)	10

5.5. Device Polling (Other)	11

5.6. Device Polling (User Counter)	11

5.7. Advanced Device Preferences	11

5.8. Basic Device Preferences	12

5.9. Address Book Preferences	12

5.10. Power Mode	13

5.11. Activation/Deactivation	13

5.12. Usage Report/Update Notification	13

6.1. Security Considerations	15

7. Stored Data	16

7.1 Stored Data	16

1. Introduction

Device Manager NX Lite is device management software which provides basic remote device management functions, such as batch configuration and device monitoring, from the comfort of the administrator’s desktop. Even though it is free software, it can still be used to manage up to 250 devices.

This document is designed to describe the product’s functioning with regards to network and data security. As Ricoh products are designed for use inside an intranet where network clients are protected by firewalls, they rely heavily on the intranet’s security policies. This document focuses on providing the information necessary to protect against potential threats from external security risks and help customers to securely incorporate the Ricoh product into their system.

# 2. System

These diagrams show the general data flow for Device Manager NX Lite.

## 2.1. Internet communication diagram

Device Manager NX Lite

UI

Web Server (Jetty)

Platform

Database (FireBird)

Intranet

Internet

RICOH Software Server

RICOH Backend Server

HTTPS

HTTP

On-line Activation/Deactivation

Send usage report.

Check for and get updates.

Proxy

(if enabled)

## 2.2. Intranet communication diagram

Device Manager NX Lite

UI

Web Server (Jetty)

Platform

Database (FireBird)

3rd Party Devices

Ricoh devices (previous generation)

Ricoh devices (current generation)

Other Ricoh devices

Ricoh-branded OEM devices

SNMP

(Printer MIB)

SNMP

(Printer MIB)

SNMP

(Printer & Private MIB)

SNMP

(Printer & Private MIB)

SNMP (Printer & Private MIB), HTTP/HTTPS FTP

SNMP (Printer & Private MIB), HTTP/HTTPS, FTP

Ricoh’s GelJet devices

The icons shown here for each device category are identical to the icons used by the Device Manager NX Lite UI.

# 3. Data Flow

## 3.1. Internet communication data flow

| Data Flow | Functions | Data |
| --- | --- | --- |
| Device Manager NX Lite -> RICOH Software Server | Activation/Deactivation | Product Key, Lock Code, License File |
| Device Manager NX Lite -> RICOH Backend Server | Usage Report | Country GUID (Product code + lock code) OS Report Date & Time Installed Date Product Name Product Version Product Option Number of devices per vendor Breakdown of Ricoh devices by generation, model type, etc. Serial numbers for up to 3 devices (Ricoh devices only, same model*) |
|  | Software Update Notification | Product Version |

*The target model is selected by choosing the device with the largest number of devices. If multiple devices of that model exist, the models with the 1st, 2nd, and 3rd largest total counters are selected for inclusion in the Usage Report.

## 3.2. Intranet communication data flow

| Data Flow | Functions | Data |
| --- | --- | --- |
| Device Manager NX Lite -> 3rd Party Devices | Collect device information. | Device’s status, supply, and counter information. No user counter. |
| Device Manager NX Lite -> Ricoh-branded OEM devices | Collect device information. | Device’s status, supply, and counter information. No user counter. |
| Device Manager NX Lite -> Ricoh GelJet devices | Collect device information. | Device’s status, toner/supply, and counter information. No user counter. |
| Device Manager NX Lite -> Other Ricoh devices | Collect device information. | Device’s status, toner/supply, and counter information. No user counter. |
| Device Manager NX Lite -> Ricoh previous generation devices | Collect device information. | Device’s status, toner/supply, and counter information. With user counter. |
|  | Basic Device Preferences | Basic device configuration. |
|  | Address book preference | Address book configuration. |
|  | Energy saving setting | Power status changes. |
| Device Manager NX Lite -> Ricoh current generation devices | Collect the device’s information. | Device’s status, toner/supply, and counter information. With user counter. |
|  | Basic Device Preferences | Basic device configuration. |
|  | Advanced Device Preferences | Advanced device configuration. |
|  | Address book preference | Address book configuration. |
|  | Power Mode preference | Power status changes. |

# 4. Access account

Device Manager NX Lite uses 3 types of access accounts to communicate with devices.

## 4.1. SNMP Access Account

The following access account is used for SNMP communication:

[Using SNMP V1/V2]

Read community (default value is “public”)

Write community (default value is “admin”)

[Using SNMP V3]

Username (default value is “admin”)

Password (default value is none)

Authentication algorithm [MD5/SHA1] (default value is “MD5”)

Encryption password (default value is none)

Encryption algorithm [DES/AES128] (default value is “DES”)

Context Name (default value is “GWNCS”)

Note: The account must have full device administrator privileges (User Administrator, Machine Administrator, Network Administrator, and File Administrator)

## 4.2. Web Access Account

This access account is used for web service (HTTP/HTTPS) communication:

Username (the default value is “admin”)

Password (the default value is blank)

Note: The account must have full device administrator privileges (User Administrator, Machine Administrator, Network Administrator, and File Administrator)

4.3. SDK access account

This access account is used for collecting SDK application information from the device.

Password (the default value is encrypted, and this is not editable.)

# 5. Protocols and Ports

## 5.1. Discovery

|  | Operation | Protocol | Port | Access Account | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | Collecting device information. (Device Manager NX Lite -> Device) | SNMP | UDP/161 | SNMP V1/V2: Read Community Or SNMP V3 access account | Devices with one of the following MIBs can be registered: - sysObjectID - prtGeneralConfigChanges - Ricoh Search Function |
| 2 | Confirming the web access account. (Device Manager NX Lite -> Device) | HTTP/SOAP or HTTPS/SOAP | TCP/80 or TCP/443 | Web access account |  |
| 3 | Collecting SDK information. (Device Manager NX Lite -> Device) | FTP and HTTPS | TCP/21 TCP/51443 | Web access account SDK access account | FTP is used to check for the SDK Platform. Disabling FTP on the device does not affect this process. |

## 5.2. Device Polling (Status)

|  | Operation | Protocol | Port | Access Account | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | Collecting device status information. (Device Manager NX Lite -> Device) | SNMP | UDP/161 | SNMP V1/V2: Read Community Or SNMP V3 access account |  |

## 5.3. Device Polling (Tray/Toner Ink)

|  | Operation | Protocol | Port | Access Account | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | Collecting device Tray/Toner Ink information. (Device Manager NX Lite -> Device) | SNMP | UDP/161 | SNMP V1/V2: Read Community Or SNMP V3 access account |  |

## 5.4. Device Polling (Counter)

|  | Operation | Protocol | Port | Access Account | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | Collecting device Counter information. (Device Manager NX Lite -> Device) | SNMP | UDP/161 | SNMP V1/V2: Read Community Or SNMP V3 access account |  |

## 5.5. Device Polling (Other)

|  | Operation | Protocol | Port | Access Account | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | Collecting device Other information, such as MAC address, etc. (Device Manager NX Lite -> Device) | SNMP | UDP/161 | SNMP V1/V2: Read Community Or SNMP V3 access account |  |
| 2 | Collecting SDK information (Device Manager NX Lite -> Device) | FTP and HTTPS | TCP/21 TCP/51443 | Web access account & SDK access account | FTP is used to check for the SDK Platform. Disabling FTP on the device does not affect this process. |

## 5.6. Device Polling (User Counter)

|  | Operation | Protocol | Port | Access Account | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | Confirming the device’s response. (Device Manager NX Lite -> Device) | SNMP | UDP/161 | SNMP V1/V2: Read Community Or SNMP V3 access account |  |
| 2 | Collecting User Counter information. (Device Manager NX Lite -> Device) | HTTP/SOAP or HTTPS/SOAP | TCP/80 or TCP/443 | Web access account |  |

## 5.7. Advanced Device Preferences

|  | Operations | Protocol | Port | Access Account | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | Confirming the device’s response. (Device Manager NX Lite -> Device) | SNMP | UDP/161 | SNMP V1/V2: Read Community Or SNMP V3 access account |  |
| 2 | Collecting preference information. (Device Manager NX Lite -> Device) | HTTP/SOAP or HTTPS/SOAP | TCP/80 or TCP/443 | Web access account |  |
| 3 | Configuring device preferences. (Device Manager NX Lite -> Device) | HTTP/SOAP or HTTPS/SOAP | TCP/80 or TCP/443 | Web access account |  |

## 5.8. Basic Device Preferences

|  | Operation | Protocol | Port | Access Account | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | Confirming the device’s response. (Device Manager NX Lite -> Device) | SNMP | UDP/161 | SNMP V1/V2: Read Community Or SNMP V3 access account |  |
| 2 | Collecting preference information. (Device Manager NX Lite -> Device) | HTTP/SOAP or HTTPS/SOAP | TCP/80 or TCP/443 | Web access account |  |
|  |  | SNMP | UDP/161 | SNMP V1/V2: Read Community Or SNMP V3 access account |  |
| 3 | Configuring device preferences. (Device Manager NX Lite -> Device) | HTTP/SOAP or HTTPS/SOAP | TCP/80 or TCP/443 | Web access account |  |
|  |  | SNMP | UDP/161 | SNMP V1/V2: Read Community Or SNMP V3 access account |  |
| 4 | Restarting the device. (Device Manager NX Lite -> Device) | HTTP/SOAP or HTTPS/SOAP | TCP/80 or TCP/443 | Web access account |  |

## 5.9. Address Book Preferences

|  | Operation | Protocol | Port | Access Account | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | Confirming the device’s response. (Device Manager NX Lite -> Device) | SNMP | UDP/161 | SNMP V1/V2: Read Community Or SNMP V3 access account |  |
| 2 | Collecting Address Book information. (Device Manager NX Lite -> Device) | HTTP/SOAP or HTTPS/SOAP | TCP/80 or TCP/443 | Web access account |  |
| 3 | Configuring the Address Book. (Device Manager NX Lite -> Device) | HTTP/SOAP or HTTPS/SOAP | TCP/80 or TCP/443 | Web access account |  |

## 5.10. Power Mode

|  | Operation | Protocol | Port | Access Account | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | Confirming the device’s response. (Device Manager NX Lite -> Device) | SNMP | UDP/161 | SNMP V1/V2: Read Community Or SNMP V3 access account |  |
| 2 | Configuring the power mode. (Device Manager NX Lite -> Device) | SNMP | UDP/161 | SNMP V1/V2: Read Community Or SNMP V3 access account |  |

## 5.11. Activation/Deactivation

Activation/deactivation is internet-based, so this communication must pass through the proxy server, if one is in use.

|  | Operations | Protocol | Port | Access privileges | Note |
| --- | --- | --- | --- | --- | --- |
| 1 | Confirm the request for Activation/Deactivation. (Device Manager NX Lite -> Ricoh Software Server) | HTTPS | TCP/443 | Retained in Device Manager NX Lite |  |

## 5.12. Usage Report/Update Notification

Usage Reports and Update Notifications are internet-based, so th

|  | Operations | Protocol | Port | Access privileges | Note |
| --- | --- | --- | --- | --- | --- |
| 1 | Transmit usage reports & check for updates. (Device Manager NX Lite -> Ricoh Backend Server) | HTTP | TCP/80 | Retained in Device Manager NX Lite | Device Manager NX Lite does not support direct downloading of update data. |

Note

-The port number cannot be changed.

-Device Manager NX Lite initiates all network communication. Also, Device Manager NX Lite has no open ports in order to prevent an unauthorized external connection.

6. General Security Considerations

## 6.1. Security Considerations

As devices do not have secure communication enabled by default, communication between Device Manager NX Lite and devices is not encrypted by default. Please configure HTTPS and SNMPv3 settings if secure protocols are required.

When Device Manager NX Lite communicates with a device via SSL/TLS communication (depends on device configuration), Device Manager NX Lite uses a certificate to encrypt the communication, but does not check the validity of the certificate. Also, importing a device certificate to the PC where Device Manager NX Lite is installed has no effect, as Device Manager NX Lite does not support custom certificates.

If a device has both HTTP and HTTPS enabled, HTTPS is used.

Device Manager NX Lite supports the following ciphers/encryption protocols:

-Hash- SHA-2 (SHA-256)

-Public Key- RSA2048

-Common Key- AES256, AES128, or 3TDEA

# 7. Stored Data

## 7.1 Stored Data

| Data | Item | Detail |
| --- | --- | --- |
| Device Data | Encryption | Not encrypted. Device Data is stored in a DB. The DB has no encryption or other protection. However, it has no ports in order to prevent external connections. The DB is created in the following directory: <Install Folder>\data\database\firebird\ Device Manager NX Lite uses a special account and password to access that DB. The account and password are encrypted using Blowfish (128-bit) and are stored in gc.properties: <install folder>\configuration\gc.properties |
|  | Back-up | Included in the System Back-up. |
|  | Log Entry | None. |
|  | Remarks | None. |
| Device Access Account Data | Encryption | Not encrypted. Access Account Data is stored in the same DB as Device Data. |
|  | Back-up | Included in the System Back-up. |
|  | Log Entry | None. |
|  | Remarks | None. |
| Configuration Data (Template/Task) | Encryption | Not encrypted. Configuration Data is stored in the same DB as Device Data. In addition, Address Book Preferences and Advanced Device Preferences are encrypted using AES (256-bit) and are stored in the repository: <install folder>\data\repository\ |
|  | Back-up | Included in the System Back-up. |
|  | Log Entry | Task results are recorded in the Task Logs. |
|  | Remarks | None. |
| System Settings | Encryption | Not encrypted. System Settings are stored in the same DB as Device Data. In addition, some information is also stored in gc.properties: <install folder>\configuration\gc.properties The license code, authentication credentials for the eDC Server, and the password of the Proxy Server account are encrypted using Blowfish (128-bit). The other settings are stored in plain text. Information stored in only the DB file: -Custom Properties -System Data Settings Information stored in both the DB file and settings file: -Activation and Usage Report Settings -Proxy Settings -Display Settings |
|  | Back-up | Included in the System Back-up. |
|  | Log Entry | Configuration of system settings are not recorded in System Logs. |
|  | Remarks | None. |
| System Password | Encryption | Not encrypted. The System Password is stored in the same DB as Device Data. |
|  | Back-up | Included in the System Back-up. |
|  | Log Entry | Changes to the system password are not recorded in the System Logs. |
|  | Remarks | Use of a blank password is supported. |
| Logs (Task, System, Notification) | Encryption | Not encrypted. The Logs are stored in the same DB as Device Data. |
|  | Back-up | Included in the System Back-up. |
|  | Log Entry | - |
|  | Remarks | None. |
| Back-up Data | Encryption | None. The Back-up Data is just a copy of the DB file. |
|  | Back-up | - |
|  | Log Entry | The result of the back-up process is recorded in the System Logs. |
|  | Remarks | None. |
| Debug Log | Encryption | Plain text. Not Masked. |
|  | Back-up | Not included in the System Back-up. |
|  | Log Entry | - |
|  | Remarks | None. |