Issued March 19th, 2013

Security White Paper

for

RICOH Smart Presenter

Solution Support Department

Service and Support Center

Global Marketing Group

Ricoh Company, Ltd.

Document version: 1.0

Notice:

This document may not be reproduced or distributed in whole or in part, for any purpose or in any fashion without the prior written consent of Ricoh Company Limited. Ricoh Company Limited retains the sole discretion to grant or deny consent to any person or party.

Copyright © 2013 by Ricoh Company Ltd.

All product names or product illustrations, including desktop images, used in this document are trademarks, registered trademarks or the property of their respective companies. They are used throughout this book in an informational or editorial fashion only. Ricoh Company, Ltd. does not grant or intend to grant hereby any right to such trademarks or property to any third parties. The use of any trade name or web site is not intended to convey endorsement or any other affiliation with Ricoh products.

Although best efforts were made to prepare this document, Ricoh Company Limited makes no representation or warranties of any kind with regards to the completeness or accuracy of the contents and accepts no liability of any kind including but not limited to performance, merchantability, fitness for any particular purpose, or any losses or damages of any kind caused or alleged to be caused directly or indirectly from this document.

Document version history:

| Version | Date of Issue | Revision |
| --- | --- | --- |
| 1.0 | March, 2013 | 1st Release |

1. Introduction	4

2. System	5

2.1. Internet Communication Diagram	5

2.2. Intranet Communication Diagram (Planned Meetings)	6

2.3. Intranet Communication Diagram (Casual Meetings)	7

3. Data Flow	8

3.1. Internet Communication Data Flow	8

3.2. Intranet Communication Data Flow (Planned Meetings)	8

3.3. Intranet Communication Data Flow (Casual Meeting)	8

4. Protocols and Ports	10

4.1. Activation Tool	10

4.2. Planned Meetings	10

4.3. Casual Meetings	10

5. Stored Data	12

5.1 Stored Data (RICOH Smart Presenter)	12

5.2 Stored Data (RICOH Conference Center)	13

# 1. Introduction

RICOH Smart Presenter is a paperless meeting solution comprised of two components: the RICOH Conference Center and the RICOH Smart Presenter app. It allows tablet displays to be used for presentation document distribution without the need to print paper copies of the documents.

There are two usage options available: with or without RICOH Conference Center.

When using RICOH Conference Center, meeting information can be registered in advance, and meeting documents can be uploaded from any PC web browser. Meeting participants can easily join a meeting and access the uploaded meeting documents, as well as make, share, and export notes by using the RICOH Smart Presenter app.

In addition, RICOH Smart Presenter can be displayed by using a wireless RICOH projector.

This document is designed to describe the product’s functions with regards to network and data security. As Ricoh products are designed for use inside an intranet where network clients are protected by firewalls, they rely heavily on intranet security policies. This document focuses on providing the information necessary to protect against potential threats from external security risks and help customers to securely incorporate the Ricoh product into their system.

# 2. System

These diagrams show the general data flow for RICOH Smart Presenter.

## 2.1. Internet Communication Diagram

Conference Center

Intranet

Internet

RICOH Software Server

On-line Activation/Deactivation

Proxy

(if enabled)

Activation

Tool

HTTPS

#1 443 (HTTPS)

## 2.2. Intranet Communication Diagram (Planned Meetings)

Meeting information

Conference Server

File Server

Web Server

#8　10389

ApacheDS

Openfire

#6　8009

Apache

Tomcat

#5　7443

#7　5432

#2  5222

#3  8443 (HTTPS)

#4　443 (HTTPS)

iPad

Client PC

PostgreSQL

#7　5432

RICOH Conference Center Server (Windows Server2008 R2)

HTTPS

HTTPS

XMPP

## 2.3. Intranet Communication Diagram (Casual Meetings)

#9  Available Port

#10  8080

iPad(Client)

#12  4352

#13  161

#12  4352

#15  2014

#16  2013

#17  2012

iPad

Projector (PJ X3340N/WX3340N

/WX4240N/WX4130N)

iPad

Projector (PJ WX3230N/X3240N

/WX3231N/X3241N)

iPad（Host）

#11  5353

#14  80

# 3. Data Flow

## 3.1. Internet Communication Data Flow

| Data Flow | # | Functions | Data |
| --- | --- | --- | --- |
| RICOH Conference Center -> RICOH Software Server | 1 | Activation/Deactivation | Product Key, Lock Code, License File, Country Information |

## 3.2. Intranet Communication Data Flow (Planned Meetings)

[iPad -> RICOH Conference Center Server]

| Data Flow | # | Functions | Data |
| --- | --- | --- | --- |
| iPad -> Conference server | 2 | Manage meetings. | Meeting list, meeting ID, password, shared page number, shared notes data. |
| iPad -> File server | 3 | Get meeting documents. | PDF files. |

[Client PC -> RICOH Conference Center Server]

| Data Flow | # | Functions | Data |
| --- | --- | --- | --- |
| Web browser -> Administrator tool, Conference Center (Meeting Management, Meeting list) | 4 | Manage planned meetings | Application used to manage, edit and reserves meetings. |

[RICOH Conference Center Internal Communication]

| Data Flow | # | Functions | Data |
| --- | --- | --- | --- |
| Web Server -> Conference Server | 5 | Control meeting information. | Meeting list, meeting ID, meeting information. |
| Web Server -> File Server | 6 | Control meeting documents. | Meeting documents. |
| Conference server, File Server -> PostgreSQL | 7 | Control meeting information. | Information to reserve/open/participate in meetings. |
| Conference server -> Apache DS | 8 | Control account information. | Account information. |

## 3.3. Intranet Communication Data Flow (Casual Meeting)

[iPad(Client) -> iPad(Host)]

| Data Flow | # | Functions | Data |
| --- | --- | --- | --- |
| iPad(Client) -> iPad(Host) | 9 | Manage meetings. | Information to open/share information/exit/participate in meetings. Meeting ID, meeting password, shared page number, shared notes data, etc. |
| iPad(Client) -> iPad(Host) | 10 | Get meeting documents. | PDF files. |
| iPad(Client) -> iPad(Host) | 11 | Get device name. | Meeting list, members list. |

[iPad -> Projector]

| Data Flow | # | Functions | Data |
| --- | --- | --- | --- |
| iPad -> Projector | 12,13,15,17 | Set/get projector information | Projector settings. Keep data connection alive. |
| iPad -> Projector | 14,16 | Project to projector | Projection image data. |

# 4. Protocols and Ports

## 4.1. Activation Tool

Activation/deactivation is internet-based, so this communication must pass through the proxy server, if one is in use.

| # | Communication Path | Protocol | Port | Encryption | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | Confirm the request for Activation/Deactivation. (RICOH Conference Center -> Ricoh Software Server) | HTTPS | TCP/433 | Y | If the port is set in [Proxy Settings], the set port will be used. |

## 4.2. Planned Meetings

[iPad -> RICOH Conference Center Server]

| # | Communication Path | Protocol | Port | Encryption | Notes |
| --- | --- | --- | --- | --- | --- |
| 2 | Access Conference Server from iPad. | XMPP | 5222 | Y | Received module: Openfire (Ver3.7.1) |
| 3 | Access meeting documents from iPad. | HTTPS | 8443 | Y | Received module: Tomcat (Ver7.0.27) |

[Client PC -> RICOH Conference Center Server]

| # | Communication Path | Protocol | Port | Encryption | Notes |
| --- | --- | --- | --- | --- | --- |
| 4 | Access Administrator tool, Conference Center (Meeting Management, Meeting list) from PCs. | HTTPS | 433 | Y | Received module: Apache (Ver2.2.22) |

[RICOH Conference Center Internal Communication]

| # | Communication Path | Protocol | Port | Encryption | Notes |
| --- | --- | --- | --- | --- | --- |
| 5 | Access Conference Server from Administrator tool or Conference Center (Meeting Management, Meeting list). | Bosh | 7443 | Y | Received module: Openfire (Ver3.7.1) |
| 6 | Access meeting documents from Administrator tool or Conference Center (Meeting Management, Meeting list). | Ajp | 8009 | N | Received module: Tomcat-Apache (Ver7.0.27) |
| 7 | Access DB from Conference Server and File server（Internal processing） | PostgreSQL | 5432 | N | Received module: PostgreSQL (Ver9.1.3-1) |
| 8 | Access LDAP server from Conference Server（Internal processing） | LDAP | 10389 | N | Received module: ApacheDS (Ver2.0.0-M7) |

## 4.3. Casual Meetings

[iPad(Client) -> iPad(Host)]

| # | Communication Path | Protocol | Port | Encryption | Notes |
| --- | --- | --- | --- | --- | --- |
| 9 | iPad(Client) -> iPad(Host) | XMPP | 49152~65535 (Available ports) | N | Access HostController |
| 10 | iPad(Client) -> iPad(Host) | HTTP | 8080 | N | Access WebDAV |
| 11 | iPad(Client) -> iPad(Host) | UDP | 5353 | N | Bonjour |

[iPad -> Projector]

| # | Communication Path | Protocol | Port | Encryption | Notes |
| --- | --- | --- | --- | --- | --- |
| 12 | iPad -> Projector | PjLink | 4352 | N | Pjlink (Projector) |
| 13 | iPad -> Projector | SNMP | 161 | N | MIB (PJ X3340N/WX3340N/WX4240N/WX4130N) |
| 14 | iPad -> Projector | HTTP | 80 | N | Projection (PJ X3340N/WX3340N/WX4240N/WX4130N) |
| 15 | iPad -> Projector | TCP | 2014 | N | Get information (PJ WX3230N/X3240N/WX3231N/X3241N) |
| 16 | iPad -> Projector | TCP | 2013 | N | Projection (PJ WX3230N/X3240N/WX3231N/X3241N) |
| 17 | iPad -> Projector | UDP | 2012 | N | Keep-alive (PJ WX3230N/X3240N/WX3231N/X3241N) |

# 5. Stored Data

## 5.1 Stored Data (RICOH Smart Presenter)

| Data | Item | Security Note |
| --- | --- | --- |
| RoomServices | Access Control | None. |
|  | Encryption | Not encrypted. |
|  | Back up | None. |
|  | Log Entry | None. |
|  | Remarks | The data of meeting list and restoration. A planned meeting password is included. Access to the data can only be made via the application. [File Path] <Application_Home>/RoomData/ |
| ConferenceData | Access Control | None. |
|  | Encryption | Not encrypted. |
|  | Back-up | None. |
|  | Log Entry | None. |
|  | Remarks | The meeting data of a held meeting. ConferenceData is deleted when a user exits a meeting. If the application crashes, ConferenceData is retained and restored when the application is restarted. |
| Meeting Documents (temp) | Access Control | None. |
|  | Encryption | Not encrypted. |
|  | Back-up | None. |
|  | Log Entry | None. |
|  | Remarks | [File Path] - Downloaded meeting documents (PDF): <Application_Home>/tmp/ConferenceData/<meetingID>/documents/ - thumbnail png: <Application_Home>/tmp/ This data is deleted when a user exits a meeting. If the application crashes, this data is retained and restored when the application is restarted. |
| Meeting Documents (data) | Access Control | None. |
|  | Encryption | Not encrypted. |
|  | Back-up | None. |
|  | Log Entry | None. |
|  | Remarks | [File Path] <Application_Home>/Documents/ Documents added when preparing a casual meeting and added during an active meeting. It can be added via Safari or iTunes. It can be added and deleted by users. |
| ServerSetting | Access Control | None |
|  | Encryption | Not encrypted. |
|  | Back-up | None. |
|  | Log Entry | None. |
|  | Remarks | The address and name of Conference Center server. This data is retained if the application is shut down. |

## 5.2 Stored Data (RICOH Conference Center)

| Data | Item | Security Note |
| --- | --- | --- |
| Meeting Data (Stored Postgre SQL) | Access Control | A fixed PostgreSQL Administrator Account/Password is used to access the PostgreSQL. The DB is protected by a password. |
|  | Encryption | Not encrypted. |
|  | Back-up | None. |
|  | Log Entry | None. |
|  | Remarks | Data for planned meetings. It will be deleted when the meeting is deleted. |
| Meeting Documents (Office) (Stored windows file system) | Access Control | Any Conference Center server user account. |
|  | Encryption | Not encrypted. Obfuscated by changing extension. |
|  | Back-up | None. |
|  | Log Entry | None. |
|  | Remarks | It will be deleted when the meeting is deleted. |
| Meeting Documents (PDF) (Stored windows file system) | Access Control | Any Conference Center server user account. |
|  | Encryption | Not encrypted. Obfuscated by changing extension. |
|  | Back-up | None. |
|  | Log Entry | None. |
|  | Remarks | It will be deleted when the meeting is deleted. |
| Administrator tool account information | Access Control | A fixed Account/Password of ApacheDS is used to access the ApacheDS. The data is protected by a password. |
|  | Encryption | The password is encrypted. |
|  | Back-up | None. |
|  | Log Entry | None |
|  | Remarks | None |
| Activation Data | Access Control | Any Conference Center server user account. |
|  | Encryption | The files are not encrypted. - License Code file: Encrypted license code is recorded. - Deactivation information file: Encrypted license code is recorded. - Activation Tool settings file: The password of the Proxy authentication is encrypted. Others are not encrypted. |
|  | Back-up | None. |
|  | Log Entry | None. |
|  | Remarks | None. |
| Log | Access Control | Any Conference Center server user account. |
|  | Encryption | Not encrypted. |
|  | Back-up | None. |
|  | Log Entry | None. |
|  | Remarks | The creation of log files is rotated according to fixed counts and the number of log files. PsgreSQL and Apache log files are rotated, but the number of files that can be created is unlimited. [File Path] - Apache: <install folder>\Log\Apache\ - ApacheDS: <install folder>\Log\ApacheDS\ - Openfire: <install folder>\Log\Openire\ - PostgreSQL: <install folder>\Log\PostgreSQL\ - Tomcat: <install folder>\Log\Tomcat\ - Stop/Start service script: <install folder>\Log\Tools\ - Activation Tool: <install folder>\Log\Tools\ActivationTool\ - Installer: <install folder>\Log\IS\ |
| Certificates | Access Control | Any Conference Center server user account. |
|  | Encryption | All files are encrypted. |
|  | Back-up | None. |
|  | Log Entry | None. |
|  | Remarks | [File Path] - Apache: <System drive>:\Program Files (x86)\RICOH\RICOH Conference Center\Apache Software Foundation\Apache2.2\conf\ssl.key <System drive>:\Program Files (x86)\RICOH\RICOH Conference Center\Apache Software Foundation\Apache2.2\conf\ssl.crt - Openfire: <System drive>:\Program Files (x86)\RICOH\RICOH Conference Center\openfire\resources\security - Tomcat: <install folder>\Apache Software Foundation\Tomcat 7.0\conf\ |