Security White Paper for

Streamline NX Secure Print Manager

Version 1.2

Solution Value proposition Section

Global Technology Support Department

Service and Support Center

Global Marketing Group

Document version

SLNXSPMV1.2.0-1.0

Notice:

This document may not be reproduced or distributed in whole or in part, for any purpose or in any fashion without the prior written consent of Ricoh Company Limited. Ricoh Company Limited retains the sole discretion to grant or deny consent to any person or party.

Copyright © 2011 by Ricoh Company Ltd.

All product names or product illustrations, including desktop images, used in this document are trademarks, registered trademarks or the property of their respective companies. They are used throughout this book in an informational or editorial fashion only. Ricoh Company, Ltd. does not grant or intend to grant hereby any right to such trademarks or property to any third parties. The use of any trade name or web site is not intended to convey endorsement or any other affiliation with Ricoh products.

Although best efforts were made to prepare this document, Ricoh Company Limited makes no representation or warranties of any kind with regards to the completeness or accuracy of the contents and accepts no liability of any kind including but not limited to performance, merchantability, fitness for any particular purpose, or any losses or damages of any kind caused or alleged to be caused directly or indirectly from this document.

Document version history:

| Version | Date of Issue | Revision |
| --- | --- | --- |
| 1.0 | August, 2011 | - Initial Release This document applies to the following products: Secure Print Manager v1.2.0.0 |

1. Introduction	4

2. System	5

2.1 System overview	6

2.1.1 Print data flow for Secure Print Manager (Server).	6

2.1.2 Print data flow for Secure Print Manager (Embedded).	7

2.1.3 Print data flow for Direct Print (without SPM).	7

2.2 Services	8

3. Network Protocols	9

3.1 Protocols and Ports	9

3.2 Recommended precautions	9

3.3 Security	10

4. Data Flow & Storage	12

4.1 Stored Data - Secure Print Manager (Server)	12

4.2 Data Access Permission - Secure Print Manager (Server)	13

4.3 To increase the number of logs retained/the max size of a log file	14

4.4 Stored Data – Secure Print Manager (Embedded)	15

4.5 Data Access Permission – Secure Print Manager (Embedded)	15

4.6 Stored Data – Direct Print (without SPM)	16

4.7 Data Access Permissions – Direct Print (without SPM)	16

# 1. Introduction

This document provides security information about Streamline NX Secure Print Manager (Server) (SPM-S), Secure Print Manager (Embedded) (SPM-E), and Streamline NX Direct Print.

Secure Print Manager (Server) (SPM-S) is a print server that can be accessed by the SDK/J application Secure Print Manager (Embedded) (SPM-E). Jobs can be submitted to the SPM (Server) and stored there. The jobs can then be accessed and printed via any MFP with the SPM (Embedded) installed. Also, the direct printing for LPs is supported via the Point & Print feature.

Other points:

- Administrators can configure Secure Print Manager (Server) to automatically delete jobs after printing.

- Alternatively, administrators can delete stored jobs manually. Users are able to delete only their own stored jobs.

- Point & Print must be used to distribute drivers from the SPM-S server to users.

# 2. System

MFP

MFP

SFM

Client

AAM

SFM

Client

AAM

SPM

SPM

Request for authentication

Send Profile Settings

#### AAM Server

#### SFM Server

From SPM Server PC to MFP

- Print Job List

- Print Job

From MFP to SPM Server PC
- JobList Request

- Job Request

Authentication

LP

LP

LP

Authentication

LDAP Server/ AD Domain Controller

#### SPM Server PC

From SPM Server PC to LP

- Print Job

MIB access

- Accounting Information

Job Information

SPM Admin Tool

Web Browser

ADM Admin Tool

Web Browser

Admin Tool Operations

#### ADM Server

Get registered servers

Single Sign On

Job/System Log

MS SQL

Server

Print Job

#### Client PC

Admin/Management Functions

## 2.1 System overview

Note: Information about AAM, SFM, and ADM are described in the corresponding Security White Papers.

### 2.1.1 Print data flow for Secure Print Manager (Server).

Secure Print with SPM (embedded SDK app):

Direct Print (with SPM):

### 2.1.2 Print data flow for Secure Print Manager (Embedded).

Secure Print Manager (Embedded)

Device Print Application

Secure Print Manager

ADM

HTTP(S)

Print data HTTP(S)

Print data

(Server)

TCP/IP 9100

### 2.1.3 Print data flow for Direct Print (without SPM).

Direct Print

Print Application

Client/Admin PC

HTTP(S)

Print data

Print data

TCP/IP 9200

AAM (S)

TCP/IP 9100

AAM (E)

Device

## 2.2 Services

Services installed or used by SPM-S:

| Service Name | Description |
| --- | --- |
| Streamline NX Secure Print Manger | The main SPM-S service. This service is installed by SPM-S. |
| Print Spooler | Spooler service in Windows printing system. This service is a part of Windows. |

# 3. Network Protocols

## 3.1 Protocols and Ports

Secure Print Manager (Server)

| Features | Service | Protocol | Port No | Purpose |
| --- | --- | --- | --- | --- |
| PrintSpooler | NetBIOS Session Service | TCP | 139 | Spooling the job |
|  | SMB | TCP | 445 |  |
| SPM-S Service | http | TCP | 8080（*1） | Administrative Tool/User Job Tool |
|  | https | TCP | 8443（*1) |  |
|  | http | TCP | 8080（*1） | Connection from an MFP/LP |
|  | https | TCP | 8443(*1） |  |
| SQL Server（StreamlineNX） | http | TCP | 1024 - 65535 (*2) | DB |

(*1) Default values: These can only be changed during installation of SPM-S.

(*2) The SQL Server automatically finds a free port to use. The port being used by SQL Server can be found in the SQL Server Configuration Manager:

Secure Print Manager (Embedded)

| Features | Service | Protocol | Port No | Destination | Description |
| --- | --- | --- | --- | --- | --- |
| Administrative Tool | http | TCP | 8080 | From Client PC/ADM to SPM (Embedded) | SPM-E configuration web page. |
|  | https | TCP | 51443 |  |  |
| Remote Print | http | TCP | 8080 (*1) | Secure Print Manager -Server | Job lists and print data transferred from the Secure Print Manager (Server) to the printer that actually prints the job. |
|  | https | TCP | 8443 (*1) |  | Job lists and print data transferred from the SPM-S to the printer via TLS. |
| Direct Print (diprint) | PDL Data Stream | TCP | 9100 (*1) | MFP/LP Print application |  |

(*1) Default values: These can only be changed during installation of SPM-S.

(*2) SPM-E can act both as a server and as a client

Direct Print

| Features | Service | Protocol | Port No | Destination | Description |
| --- | --- | --- | --- | --- | --- |
| Direct Print Service | PDL Data Stream | TCP | 9200 | From client PC to Print Service |  |

## 3.2 Recommended precautions

Secure Print Manager uses TLS v1.0 when HTTPS is enabled.

## 3.3 Security

Connection between the SPM-S server and an MFP/LP (initiated by the MFP)

- Secure Print with SPM

HTTP or HTTPS can be used to connect to the SPM-S server from an MFP/LP. HTTPS can be enabled by ADM.

- If HTTP is used, the job information is not encrypted. If security is a concern, HTTPS is recommended.

Note: Passwords are not transmitted between the MFP and SPM-S.

- Direct Print (with SPM) to Laser Printer (LP)

When using Direct Print for LPs, the Secure Print Manager server (SPM-S) sends the print job data to the LP’s TCP/IP port 9100.

SPM-S obtains the number of printed pages from an LP using SNMP. SPM-S continues monitoring the status of the LP through polling (SNMP) until the print job is completed. Then SPM-S registers the collected information on the number of output pages to the AAM server.

Note: The supported versions of SNMP are v1 and v2.

- Collecting and registering the Accounting information.

| No. | Purpose | Protocol | Port | Remarks |
| --- | --- | --- | --- | --- |
| 1 | Retrieving the accounting information | SNMP V1/V2 | 161 | The following item is necessary: 1. Read community name |
| 2 | Registering the accounting information | HTTPS | Port on AAM-S | Access to AAM-S. |

Using HTTPS may cause a decrease of printing performance. The degree of the decrease (if any) depends on the job and the model.

# 4. Data Flow & Storage

## 4.1 Stored Data - Secure Print Manager (Server)

| Data type | Security |
| --- | --- |
| Print data | - Stored in the SPM-S file system. - Encrypted (AES 128-bit). Decrypted before being sent to an MFP/LP. - Delete job after printing: A print job can be deleted automatically after printing is complete. This can be configured by administrators using the web interface’s "Delete job after printing" setting. - Auto Delete Interval: Jobs are stored on the SPM-S server for a certain period of time (configurable) and then automatically deleted once that time has elapsed. This can be configured by administrators using the web interface. This can be used in conjunction with the "Delete job after printing" setting to automatically delete unprinted jobs. - Administrators can manually delete any job via the web interface. - Users can manually delete any of their own jobs via the web interface or the operation panel. |
| Job Attributes | - Various attributes about each job are stored in the database on the SPM-S server. - These attributes (date, user name, number of pages, etc.) are extracted from submitted jobs and stored in the database. They are then used to display the Job List on the MFP. Note: Passwords are not stored. - Job attributes are deleted when the corresponding job is deleted. Job Attributes are stored in the database unencrypted. |
| SPM-S System Configuration | - Stored both in the SQL database and the file system. - The Web Admin Tool Administrator’s password and the password for the database connection are stored encrypted (AES128). |
| Logs | - Stored in the Windows file system. - Access Log: Access to SNX SPM port and SPM-S service are logged. Administrators can download 2 days worth of Access log entries via the web UI. Service engineers can download the whole Access log. - Error Log: Errors affecting the services “SNX SPM Port” and “SPM-S Service” are logged. Only service engineers can download the Error log. - A new log file is created everyday and will be used for logging for the next 24 hours or until the size of the file reaches 10MB. If the log file reaches 10MB a new file will be created and used for the remainder of the 24 hour period or until that file reaches 10MB. - The most recent 20 log files will be retained. Older files are overwritten. Logs are not encrypted. |

*1) For information about changing the max size of a log file, please see section 4.3.

*2) For information about changing the number of log files retained, please see section 4.3.

## 4.2 Data Access Permission - Secure Print Manager (Server)

| Data type | Account Type | Permissions | Process |
| --- | --- | --- | --- |
| Print data | User | Add |  |
|  | SNX SPM Port | Add | - Encrypt and store the print data on the SPM-S hard disk. - Writes job attributes to the DB. |
|  | SPM-S Service | Delete | - "Delete job after printing" - "Auto Delete Interval” |
|  | Administrator | Delete | - Manual deletion via web interface |
|  | User | Delete | - Manual deletion via web interface or the operation panel |
| System Configuration | Administrator | Edit | - Settings changed via web interface |
|  | SPM-S Service | Edit | - Settings changed via web interface (above) are written to the database. |
| Logs | SNX SPM Port | Add Delete | - Creates and manages logs. |
|  | SPM-S Service | Add Delete | - Creates and manages logs. |
|  | Field service engineer | Download | - Any existing logs - Both the Access and Error logs relating to the SPM-S port and service. |
|  | Administrator | Download | - Access logs only (most recent 2 days). - Access logs relating to the SPM-S port and service. |

## 4.3 To increase the number of logs retained/the max size of a log file

A. SNX SPM Port

- Navigate to "Administrative Tools" > "Services" and stop the "Print Spooler" service.

- Open <install path>\jetty\webapps\spm-s\WEB-INF\conf\port_log4j.properties and edit the values shown in blue:

Access logs:

Number of logs retained	log4j.appender.port_access.MaxBackupIndex=20

Max file size 		log4j.appender.port_access.MaxFileSize=10MB

Error logs:

Number of logs retained	log4j.appender.port_error.MaxBackupIndex=20

Max file size		log4j.appender.port_error.MaxFileSize=10MB

- Navigate to "Administrative Tools" > "Services" and start the "Print Spooler" service.

B. SPM-S Service

- Navigate to "Administrative Tools" > "Services" and stop the "Streamline NX Secure Print manager" service.

- Open <install path>\jetty\webapps\spm-s\WEB-INF\conf\server_log4j.properties and edit the values shown in blue:

Access logs:

Number of logs retained	log4j.appender.server_access.MaxBackupIndex=20

Max file size		log4j.appender.server_access.MaxFileSize=10MB

Error logs:

Number of logs retained	log4j.appender.server_error.MaxBackupIndex=20

Max file size 		log4j.appender.server_error.MaxFileSize=10MB

- Navigate to "Administrative Tools" > "Services" and start the "Streamline NX Secure Print manager" service.

## 4.4 Stored Data – Secure Print Manager (Embedded)

| Data type | Security |
| --- | --- |
| System Configuration | The settings for SPM-E are stored in the device HDD. In addition, the admin password is encrypted using DES. The configuration file can be imported only by administrator. The configuration file is always encrypted. |
| Log | Administrator can import and export the logs via administrative tool. The log files are not encrypted. The total size for logs is 2MB. |

## 4.5 Data Access Permission – Secure Print Manager (Embedded)

| Data type | Account type | Event | Process |
| --- | --- | --- | --- |
| System Configuration | System | Add | System creates the initial configuration after installation. |
|  | Administrator (*) | Read Modify | System downloads and modifies the configurations via the administrative tool. |
| Log | System | Add Delete | System adds and deletes the logs. |
|  | Services Engineer (*) | Read | Services Engineer downloads the log files via administrative tool. |

(*) The use of SSL is recommended whenever possible.

## 4.6 Stored Data – Direct Print (without SPM)

| Data type |  | Security |
| --- | --- | --- |
| System Configuration | Listening port number, Destination port number | The settings for Direct Print are stored in the SD card. |

## 4.7 Data Access Permissions – Direct Print (without SPM)

| Data | Account Type | Event | Process |
| --- | --- | --- | --- |
| System Configuration | System | Read |  |